| Client order no. | Item-No. | Offer no. |
| --- | --- | --- |
| Order no. | Consignment no. | Project |
| Remarks |  |  |

Safe Area

| Electrical data Branding disclaimer applies U [V] | Δ / Y | f [Hz] | P [kW] | P [hp] |  | I [A] |  | n [1/min] |  | M [Nm] | η 3) |  |  | cosφ 3) |  |  | IA/IN II/IN | MA/MN TI/TN | MK/MN TB/TN | IE-CL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | 4/4 | 3/4 | 2/4 | 4/4 | 3/4 | 2/4 |  |  |  |  |
| DOL duty (S1) - 155(F) to 130(B) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 400 | Δ | 50 | 3.00 | -/- |  | 5.90 |  | 2920 |  | 9.8 | 87.1 | 87.2 | 85.9 | 0.84 | 0.78 | 0.66 | 10.2 | 4.4 | 4.6 | IE3 |
| 690 | Y | 50 | 3.00 | -/- |  | 3.45 |  | 2920 |  | 9.8 | 87.1 | 87.2 | 85.9 | 0.84 | 0.78 | 0.66 | 10.2 | 4.4 | 4.6 | IE3 |
| 460 | Δ | 60 | 3.45 | -/- |  | 5.80 |  | 3515 |  | 9.4 | 87.5 | 87.5 | 86.4 | 0.86 | 0.81 | 0.70 | 11.0 | 4.2 | 4.8 | IE2 |
| 460 | Δ | 60 | 3.00 | -/- |  | 5.10 |  | 3530 |  | 8.1 | 88.5 | 88.1 | 86.3 | 0.83 | 0.77 | 0.65 | 12.4 | 4.9 | 5.6 | IE3 |
| IM B3 / IM 1001 |  |  | FS 90 L |  |  |  | IP55 |  |  |  | IEC/EN 60034 |  | IEC, DIN, ISO, VDE, EN |  |  |  |  |  |  |  |
| Environmental conditions : -20 °C - +40 °C / 1000 m |  |  |  |  |  |  |  |  |  |  |  | Locked rotor time (hot / cold) : 5.20 s ¦ 6.90 s |  |  |  |  |  |  |  |  |

Mechanical data

| Sound level (SPL / SWL) at 50Hz¦60Hz |  |  | 71.0 / 78.0 dB(A) 2) 3) | 76.0 / 83.0 dB(A) 2) 3) |
| --- | --- | --- | --- | --- |
| Moment of inertia |  |  | 0.0030 kg m² |  |
| Bearing DE ¦ NDE |  |  |  |  |
| Bearing lifetime |  |  |  |  |
| L10mh FRad min for coupling operation 50¦60Hz 1) |  |  | h | h |
| Regreasing device |  | Without |  |  |
| Grease nipple |  |  | -/- |  |
| Type of bearing |  |  | Preloaded bearing DE |  |
| Condensate drainage holes |  |  | Without |  |
| External earthing terminal |  |  | Without |  |
| Vibration severity grade |  |  | A |  |
| Thermal class |  |  | F |  |
| Duty type |  |  | S1 |  |
| Direction of rotation |  |  | bidirectional |  |
| Frame material |  |  | aluminum |  |
| Net weight of the motor (IM B3) |  |  | 20 kg |  |
| Coating (paint finish) |  |  | Standard paint finish C2 |  |
| Color, paint shade |  |  | RAL7030 |  |
| Motor protection | (B) 1 PTC thermistor - for tripping (2 terminals) |  |  |  |
| Method of cooling |  |  | IC411 - self ventilated, surface cooled |  |
| Carbon footprint (without options) |  |  | 97kg |  |

Terminal box

| Terminal box position | top |
| --- | --- |
| Material of terminal box | Aluminium |
| Type of terminal box | TB1 E00 |
| Contact screw thread | 3xM4 |
| Max. cross-sectional area | 4.0 mm2 |
| Main cable entry | 1xM25x1.5 |
| Main cable gland | 1 plug |
| Auxiliary cable entry | 1xM16x1.5 |
| Auxiliary cable gland | 1 plug |

Special design

| Q02 | Anti-condensation heating for 230 V (2 terminals) |
| --- | --- |

Additional information:

| Space heaters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Technical data: | 1-phase, 230 V 25W |  |  |  |